Mini-Proj           FPGA-Based Alarm Clock with snooze          Date:5/7/2025

ID#: 202323010                           Name: REDHA ALTURAIK

ID#: 202323090                           Name: MOHAMMED ALFARAJ

- Objective:

  - Learn and apply Hardware Description Languages (HDL) like Verilog.

  - Design and implement digital logic circuits.

  - Create and manage finite state machines for alarm and snooze control.

  - Use clock division techniques for accurate time tracking.

  - Interface with hardware components such as 7-segment displays, buzzers, and push buttons.

  - Apply debouncing techniques for mechanical button inputs.

  - Implement custom logic for snooze functionality.

- Methodology:

Task-1: Clock Display and Timing Logic:

- We first began by creating the timer of the clock. It should start at 00:00:00 and should continue to add up until reaching 23:59:59, which then it would go back to the initial value. To do that we had to create certain modules to count to certain values to recall the time. The project will be done using Verilog hardware programming language.

- So, we created mod10, mod6, mod3, and hoursMod10 to serve as the seconds, minutes, and hours of the project. Each mod would have the same input and output values except for hoursMod10. They would have clk, reset, load, 4-bit load_data, and CE as inputs. And 4 bit register counter and CEO as outputs. hoursMod10 has an additional input value as hoursLimit to ensure that the ones of the hours don’t go over the 23:59:59 limit when the tens of the hours is 2. Load, and load_data are used for task 2, and can be ignored and not be placed in the project until then.

- As for the body of the modules, it will contain an always block with posedge clk or posedge reset. Inside of the always block, there is an if statement that would check if certain conditions are met. The first part is to check if reset is clicked, if so, the counter inside each module would go back to 0. The next part is an else if statement that checks if load is clicked to set the timer based on the switches turned on in the FGA board which represent BCDs, and another else if to check if CE (Clock Enable) is on or not, if on, it will go to a nested if statement and check if the counter has reached the maximum value, the maximum value for each module is, 9 for mod10, 5 for mod 6, 2 for mod3, and 3 or 9 for hoursMod10 determined by the tens of the hours. If it has not reached the maximum value, it will increment the counter by 1, or else it will set it back to zero. Last is a CEO to check if the maximum value is reached.

- After that, we added the onehz and oneKhz modules to the project for the speed of the clock. Onehz counter is set to eight 9s and one khz to five 9s. and when CEO is turned on it will set it back to zero.

- Now we created a module AlarmClock use each module in project to create the timer.  And using a seven-segment display to allow the values to show up in the FGA board with wire values like D0 to D7 and the 27-bit counter, we are able to create the timer. Mod10 would be used for the ones of the seconds and minutes, Mod6 will be used for the tens of the seconds and minutes, hoursMod10 will be used for the ones of the hours, and mod3 will be used for the tens of the hours with onehz and oneKhz for the speed of the project controlled with a switch. Figures 1 to 6 shows the mods and hz modules used for the project.

- After that, we give each input and output each site value in order to test the board out and see if it works as intended. Figure 12 shows the inputs and outputs.

- After programming the board, we turned the CE value on to see if it adds 1 the counter and if it increments correctly. The seconds ones should go to 9, then back to 0 and increment the tens of the seconds to create the seconds correctly, once the tens of the seconds reach 5 and the ones reach 9, both will turn to 0 and the minutes ones will increment by 1. The minutes’ function the same as the seconds in term of logic, but they will increment the hours ones after reaching the tens of both seconds and minutes reach 5 and ones reach 9. The ones of the hours will go up to 9 as long as the tens is not equal to 2. Once they are 2, the ones will count up to 3 and then go to 0. One the time reaches 23:59:59, the timer should go back to 00:00:00. After testing it out, we found that it works as intended. Also, when reset is clicked, all values will go to 0.

Task-2: Setting Time Functions:

- After checking that the timer work correctly, we then moved to the timer setter part. The idea is to use 4 switches for the minutes and the ones of the hours, while the tens of the hours will only use 2 switches, and a button is required to set the timer.

- So, we added some input values for the AlarmClock module, which are load and 14bit load_data to hold the switch values determined if they are on or not, and load to serve as a setter when clicked.

- After that, we created wires that will hold the switches values userMinOnes, which holds the first 4 switch values, userMinTens which hold the second 4 switches, and userHoursOnes to hold the third 4 switches, and lastly, userHoursTens to hold two zeros (00) and the last two switches. The userHoursTens will have a max value of 3 and the other 15, which is unacceptable, so we created 4 more wires for conditioned values. For all the values except for userHoursTens, it will check if the user value is over 9 or not, if so, it will set 9 as the value for that number, and as for userHoursTens, it will check if it’s over 2 or not, and choose 2 if the value is over. If the value is not over the limit, it will take that value.

- After finishing the wires, add them to the other modules by modifying the mods. The load and load_data will serve as the inputs that will be used for the alarm. Load is the button that will set the values, and load_data will hold that value, and once load is clicked, it will set the value of the timer based on the on switches. As for the AlarmClock module when it comes to using the other mods, give the minutes, and hours ones, and tens to the corresponding values, the seconds will be set to 0.

- After that, we check the implementation again by adding the sites of the new inputs and programming the FGA board. The results turned out wall. When turning on the switches in this order 01 0010 0011 0000 and clicking load, it will set the timer to 12:30:00. And thanks to the condition we add for the switches, when turning all of them on, it will set the time to 23:59:00. So, the setting part of the project works correctly.

Task-3: Alarm and Snooze Logic:

- First, we began by using the wires of the conditioned setter values for the Alarm. The alarm relies on the switches similar to the setter, and a button for setting the alarm. The alarm will require wires and an input value called setAlarm.

- So, we created an always block with an if statement to check if the alarm button is clicked. When it is clicked, it will store the values of the turned-on switches inside the 4-bit alarm variables the hold the minutes and hours. These values are saved even if the switches are changed, and will change only when the setAlarm button is clicked. The tens of the hours of the alarm variable is set to 9 in the beginning of the programming to prevent it from turning on the second the project is programmed.

- Then, we added another always block to check if the time is reached. To do that we created a wire called checkAlarm, which checks if the D2 to D5 values match the alarm time. If so, it will turn to 1. With that, we created an if statement to check if reset is clicked and if check Alarm is on or off. We also created a new output for the module called AlarmLED which represents an LED light on the FGA board that will turn on when the checkAlarm is one. So, the if statement checks if reset is clicked or checkAlarm is off, it will turn the AlarmLED off, and an else if to check if it’s on, which will turn the AlarmLed on. In theory, this will keep the LED light on for 1 minute since it will turn on when the minutes and hours are equal to the alarm set time. And with that, the alarm is finished.

- Next is the snooze, which will add more time to the alarm one it is on. For the project it is required to add 5 minutes when the snooze button is clicked and the AlarmLED is on, and we added another output named snooze to the AlarmClock module.

- So, in a previous always block, we added an else if to check if both snooze is clicked and AlarmLED is on, and when so it check if adding to ones of the minutes will exceed 9 or not, if no, it will add it normally, else if will add to the hours of the minute and mod the ones of the minute to ensure it accurate timing. The same is done with hours of the minute, if less than 6, add 1, else, set it to zero, check the values of hours, if hours are 23, set them both to zero, else if, the ones are under 9 and both don’t equal 2, hours ones incremented by 1, else set the ones to 0 and tens incremented by 1.

- After that, we all the inputs and outputs their sites in order to test them and see if the work. And after programming the circuit, we did some test cases. We set the Alarm by turning on some switches and clicking setAlarm, and then turned the clock on and waited for the alarm to hit, once it hit, the LED light turned on and stayed on until the next minute. Another case was then the alarm is on, we clicked the snooze button, and check if the time is extended. And we confirmed the snooze works accordingly, and the LED light will turn on after some time, and can be extended when the alarm is on, and the alarm can be changed using the switches again. Also, the alarm stays off until a time is set for it. Figures 7 to 11 shows the whole code for the project.

- Problem faced:

- We faced a problem with the snooze, when the snooze is clicked, instead of it extending the time by 5 minutes, it extends it by 10. We tried many ways, and looked at the code to see if we can find the problem, but we couldn’t.

- A possible solution is to add a way to reduce the ones of the minute by 5 to the end of the if statement of the check snooze and AlarmLed, it will remove 5 minutes and that might fix the problem, but without the FGA board, we cannot confirm. The code will not contain the change.

- Work distribution:

| Names | Task 1 | Task 2 | Task 3 | Task 4 | Lab Report |
| --- | --- | --- | --- | --- | --- |
| REDHA ALTURAIK | 50% | 50% | 50% | 50% | 50% |
| MOHAMMED ALFARAJ | 50% | 50% | 50% | 50% | 50% |

Table 1: Work distribution for the project and report

- Results and Discussion:

The figure below contains the mod10 used for ones of the seconds and minutes, it will count up until 9 then go back to 0.

Figure 1: Mod10 for the ones of the seconds and minutes

The figure below contains the mod6 used for tens of the seconds and minutes, it will count up until 5 then go back to 0.

Figure 2: Mod6 for the tens of the seconds and minutes

The figure below contains the hoursMod10 used for ones of the hours, it will count up until 9 then go back to 0 if the input hoursLimit is 9, else, if it is 2, it will count up to 3 then 0 afterwards.

Figure 3: HoursMod10 for the ones of the hours

The figure below contains the mod3 used for tens of the hours, it will count up until 2 then go back to 0.

Figure 4: Mod3 for the tens of the hours

The figure below is for onehz, it is used to allow the counter to add up at normal speed for the alarm for normal usages.

Figure 5: onehz for the normal speed of the clock

The figure below is for oneKhz, it is used to allow the counter to add up at a faster speed than the onehz to allow for better and quicker testing than onehz.

Figure 6: oneKhz for faster speed to test the clock better

The figure below is for the AlarmClock module which features the inputs and outputs of the program. It also shows the wires used for the setting process of the circuit, speed control of the circuit, and if the hours tens is 2 or not.

Figure 7: AlarmClock code 1

The figure below is for the AlarmClock module which shows the always block in charge of setting the alarm and a part of the snooze calculation.

Figure 8: AlarmClock code 2

The figure below is for the AlarmClock module which continues the always and shows the midway code for the snooze.

Figure 9: AlarmClock code 3

The figure below is for the AlarmClock module which shows the always block showing the end of the snooze, and the checkAlarm wire, and the second always block in charge of the alarmLED

Figure 10: AlarmClock code 4

The figure below is for the AlarmClock module which shows the always block the end of the Alarm LED code and shows the mods used for the project, including the seven segment display, and onehz, and oneKhz.

Figure 11: AlarmClock code 5

Figure 12: input and output sites for the project

Conclusion

The FPGA-Based Alarm Clock with Snooze project successfully demonstrates the design and implementation of a real-time digital clock system using hardware description techniques. Through this project, key digital design concepts such as clock division, counters, binary-coded decimal (BCD) conversion, and modular system integration were applied. The clock displays the time accurately in HH:MM:SS format and allows users to set both the current time and alarm time using switches and push buttons. The alarm feature is enhanced with a snooze function, adding practical value to the system. Additionally, incorporating frequency selection and button debouncing improved the reliability and testability of the design. Overall, the project provided a strong hands-on experience in digital circuit design, FPGA programming, and real-world hardware implementation.